| Context and Background | PO Box 299 Oberon NSW 2787 ABN 98 107 506 208 | Version | Author | Reviewed | Reference |
| --- | --- | --- | --- | --- | --- |
|  |  | A-Rev 2 | SM | VP | RM-101 |
|  |  | Last Revised Date: 8 January 2014 |  |  |  |
|  |  | # OBERON YARD MAINTENANCE |  |  |  |

Introduction

OTHR is comprised of volunteers undertaking the restoration of the infrastructure of the Oberon Tarana Branch line with the view to operating a heritage tourist railway in the future. Working bees are held generally once a month when volunteer fettlers carry out a range of tasks to this aim.

Scope

This risk assessment considers all the general fettling activities that are performed within the Oberon Railway Yard precinct bounded within the OTHR lease agreement,

Personnel & Responsibilities

Fettlers operating within the Oberon yard are under the supervision of an OTHR committee member (usually the Track Manager or Assistant Track Manager)

Infrastructure involved

The railway yard holds several buildings and sheds, including the station building, coal shed, ablution block and rolling stock shed. There are tracks within the station yard and several sets of points.

Rolling Stock

The Oberon yard is currently home to all of OTHR’s rolling stock in a storage capacity including diesel locomotives, heritage passenger carriages, and a rake of heritage trucks.  OTHR does not currently operate any of these items.

OTHR does however have accreditation to operate track maintenance vehicles for the purposes of track maintenance, and presently the only item operated is a hand pushed sleeper trolley with an automatic braking system.

Public/Spectator risks

The Oberon Railway yard is private property with no or controlled public access.  The area is fenced and secured when no working bee is operating.  Public access is only invited on special open or festival days (generally twice a year).

Passenger risks

No personnel are generally permitted in any rail vehicles

SMS References

Details of other documents within OTHR SMS that are to be read in conjunction with this document (include hyperlink to website if appropriate):

- RM-001 Risk Management Procedure

- RM-002 Risk Ranking Matrix

- Toolbox meetings: TB-015, TB-013, TB-018

Development

This risk register was initially developed by a team working under Systems Manager Rick Fletcher on 15/7/2009. The team consisted of OTHR fettlers Peter Culley (TM) Harry Cole, Arthur Robinson, Bob Wilson,Richard Webb, Clive Payne, Graham Williams, Ross Allen, Charles Hazelwood and Russell Merriman

| Risk No. Hazard What can go wrong?; Risk What could happen?; Inherent Risk; Controls How could it be fixed?; Residual Risk; Finding: Issues? Problems? Adopt?; Action. L’hd.; Cons; Risk; L’hd.; PEC; Risk; By; When. |  |
| --- | --- |
| - 1 | Holes Uneven surfaces Tripping hazards. | Trips, falls and injury | 3 | 4 | 7 | ▪ Fill holes. ▪ Level uneven surfaces. ▪ Induction training. ▪ Isolate danger zones with fences or tape. | 1 1 2 1 | 1 1 4 4 | 2 2 6 5 | Adopt (done) Progressive (large area) Revise by new Toolbox mtg Start with ground controls and frames | TM TM SM TM |  |  |
| - 2 | Frame Levers, Signal Levers and Ground controls for Points. | Pinch points and entrapment if unauthorised movement. | 3 | 4 | 7 | Isolate by locking each control lever. | 1 | 1 | 2 | Adopt | tm |  |  |
| - 3 | Platform: Edge horizontal alignment. | Fouls Rollingstock. | 3 | 4 | 7 | Realign edge. | 1 | 1 | 2 | Adopt prior to running Rollingstock. | tm rm |  |  |
| - 4 | Platform: Edge timbers rotten & sagged. | Fall and injury. | 4 | 4 | 8 | ▪ Replace or invert edge timbers. ▪ Paint warning stripe on platform edge. | 2 2 | 2 2 | 4 4 | Height risk at platform. Adopt – same problem exists at all NSW platforms. | im tm |  |  |
| - 5 | Platform edge low leading to excessive slope down to edge. | Stroller or rolling object falls to track. | 3 | 4 | 7 | ▪ Raise platform edge and regrade surface. ▪ Warnings & control staff on running days. | 2 1 | 3 2 | 5 3 | Adopt with other controls Implement when operational. | im om |  |  |
|  | Platform: no passenger barrier at ends. | Injury to public when running Rolling stock. | 2 | 4 | 6 | Erect barrier fence at platform ends. | 1 | 1 | 2 | Adopt prior to running Rollingstock. | tm im |  |  |
| - 6 | Loading bank platform – low end. | Fall and injury. | 3 | 4 | 7 | Erect fence on S end & paint warning stripe on platform edge. | 2 | 3 | 5 | Adopt – same problem exists at all NSW platforms. | tm |  |  |
| - 7 | Loading bank (wool) platform – high end (1.9m) | Fall and serious injury. | 3 | 4 | 7 | ▪ Erect fence along to low level platform. ▪ Erect warning tape barrier | 1 1 | 2 3 | 3 4 | Adopt in long term. Implement in short term | im tm |  |  |
|  | Station Building – possible lead paint issue with peeling ceiling paint in most offices. | Health hazard from dust and particles | 3 | 4 | 7 | ▪ Test for lead content. ▪ Carry out appropriate OH&S procedures to remove hazard. | 1 | 1 | 2 | This may be an issue for ARTC as the building owner. |  |  |  |

| Risk No. Hazard What can go wrong?; Risk What could happen?; Inherent Risk; Controls How could it be fixed?; Residual Risk; Finding: Issues? Problems? Adopt?; Action. L’hd.; Cons; Risk; L’hd.; PEC; Risk; By; When. |
| --- |
|  | Station Building – possible asbestos issue in external cladding with asbestos sheeting (gables and rear wall). | Health hazard if disturbed. | 3 | 4 | 7 | Do not disturb painted surfaces which are in good condition. | 1 | 1 | 2 | Adopt non disturbance policy. Monitor and re-assess if repainting required. | im |  |
|  | Station Building – possible asbestos issue in external roof cladding with asbestos shingles. | Health hazard and possible contaminant for rain water tanks. | 3 | 4 | 7 | ▪ Test for asbestos content. ▪ Replace shingles and other roof cladding with environmentally safe products. ▪ Mark tank taps as unsafe for drinking. | 1 2 | 1 2 | 2 4 | This may be an issue for ARTC as the building owner. The taps may need to be locked. | im |  |
|  | Station Building – fire hazard. | The building has a timber floor and timber framed roof with combustible furnishings offering a fire risk. | 3 | 4 | 7 | ▪ Appropriate approved fire extinguishers need to be supplied and fitted along with a posted evacuation plan. ▪ Implement fire and evacuation drills. ▪ Fit smoke alarms | 2 1 1 | 3 2 2 | 5 3 3 | Adopt ALL measures to minimise risk to ALARP. | im sm im |  |
|  | Oberon Yard precinct - fire hazard. | There is a future risk of fire in the yard due to vegetation re-growth and accumulation combustible materials used in restoration of the precinct. | 3 | 4 | 7 | ▪ Develop a plan with the Town Fire Brigade and RFS to allow effective brigade access and location of hydrant points in an emergency event. ▪ Eliminate dangerous vegetation growth by constant site maintenance. ▪ Minimise the accumulation of other combustible materials. | 2 2 2 | 2 3 3 | 4 5 5 | Adopting ALL measures will achieve risk minimisation to ALARP. | ceo im tm |  |

| Risk No. | Hazard What can go wrong? |  | Risk What could happen? | Inherent Risk |  |  | Controls How could it be fixed? | Residual Risk |  |  | Finding: Issues? Problems? Adopt? | Action |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | L’hd. | Cons | Risk |  | L’hd. | PEC | Risk |  | By | When |
|  |  | Oberon Yard precinct – identification and removal of hazardous materials. | Health and worker safety hazards due to an accumulation of hazardous materials on site. | 3 | 3 | 6 | ▪ Monitor and identify hazardous materials to eliminate build-up. ▪ Develop a site Hazardous Materials Policy. | 2 2 | 2 2 | 4 4 | Adopt Adopt | im tm sm |  |
|  |  | Public Access to view operations on “open days”. | Risk of trips, falls, injury, due to straying into dangerous areas. | 3 | 3 | 6 | ▪ Education of entrants by signage and verbal directions. ▪ Erect temporary warning tape barriers. ▪ Erect permanent barriers where possible. | 2 2 1 | 2 2 2 | 4 4 3 | ALARP achieved by a combination of all 3 controls. | sm im tm |  |
|  |  | General site security. | Risk of vandalism causing theft, economic damage, damage to property and environmental damage. | 2 | 4 | 6 | ▪ General site security with man-proof perimeter fences (in place). ▪ Proximity to houses offering observation. ▪ Security cameras. ▪ Security Patrols | 2 2 2 2 | 4 4 3 4 | 6 6 5 6 | ALARP achieved by a combination of all 4 controls (fencing is in place). The cost of the last two controls may inhibit implementation. (action by Cttee of Mgt) | CM |  |
|  |  | Levelling of NW section of site by Oberon Council. | Risk of injury to OTHR volunteers. | 3 | 4 | 7 | Site to be closed except for Managers invited by Council and operations to be controlled by Council. | 1 | 1 | 2 | Adopt | tm |  |
|  |  | Runaway or accident due to sleeper trolley being propelled (manually) at a faster than necessary rate. | Risk of injury to OTHR volunteers. Risk of collision and damage to property | 3 3 | 4 4 | 7 7 | ▪ Automatic braking system is checked and operational. ▪ Trolley always operated at slow walking pace. | 2 1 | 2 1 | 4 2 | Always operate at slow walking pace - but check valid operation of braking system. | all |  |
|  |  | Potential hazard: addition of operating rollingstock. | OTHR will be slowly commencing to relocate rolling stock to the site after the yard accreditation is granted. OTHR recognises that there are potential additional hazards and risks and the organisation is currently developing specific assessment tools for each item of rollingstock prior to its relocation. |  |  |  | Risk assessments will be submitted to ITSRR for this process as soon as items are selected or allocated. |  |  |  |  |  |  |

SIGNATURE SHEET

| Risk No. | Hazard What can go wrong? | Risk What could happen? | Inherent Risk |  |  | Controls How could it be fixed? | Residual Risk |  |  | Finding: Issues? Problems? Adopt? |  | Action |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | L’hd. | Cons | Risk |  | L’hd. | Cons | Risk |  |  | By | When |  |
| ALL | As described above. | I have discussed and understand the risks. |  |  |  | I will implement the Controls described above to the best of my ability so that the Risk is managed so that it is AS LOW AS REASONABLY PRACTICAL. |  |  |  |  | Note any issues you have with the Risks, Management or Controls. |  |  |  |
| / / ________________________ [DATE OF OPERATION] OPERATING CREW. The operating crew on the day are requested to sign this document indicating that they have READ and UNDERSTOOD the RISKS and CONTROLS associated with this operation. |  | NAME (Print) ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… |  |  |  | POSITION (Task) ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… | signature ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… |  |  |  | COMMENT ………………………………………. ………………………………………. ………………………………………. ………………………………………. ………………………………………. ………………………………………. ………………………………………. ………………………………………. ………………………………………. ………………………………………. ………………………………………. ………………………………………. |  |  |  |