ACTION BY KEY: CEO= President, OM= Operations Manager, SM= Systems Manager, SEC= Secretary, PO= Public Officer, TM= Track Manager includes ATM= Assistant TM, FC= Finance Co-ordinator, TR= Treasurer, IM= Infrastructure Manager, IA= Internal Auditor, RM=Rollingstock Manager

Risks in OTHR operation at: Oberon Yard - Crane Unloading of CPH13     Affects objective: Safely unload rollingstock & move to storage area.
Document Date: 14/11/2009 Reviewed : 08/09/12	Drafted by:  SM         Assessed by: …………………..………

| Risk No. | Hazard What can go wrong? | Risk What could happen? | Inherent Risk |  |  | Controls How could it be fixed? | Residual Risk |  |  | ALARP | Finding: Issues? Problems? Adopt? | Action |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Lik. | Cons | Risk |  | Lik. | Cons | Risk | ? |  | By | When |
| 1 | CRANE Unqualified and/or uninsured operator. | Injury or death to personnel and property damage. | 3 | 5 | 8 | Independent check by OTHR on qualifications, currency and insurance. | 2 | 1 | 3 | y | Adopt. File copies with DC. | CEO SEC dc | Lift -2 week |
| 2 | CRANE Unsatisfactory access for uplift. | Damage to crane or property. | 4 | 3 | 7 | Pre-inspection of site by OTHR & crane rep | 2 | 1 | 3 | y | Adopt. CEO will be the crane operator. | CEO tm RM | Lift -1 week |
| 3 | CRANE Unsafe footing due to underground services, trenches or embankments. | Crane may be damaged or topple causing injury to crew or bystanders. | 4 | 3 | 7 | Check with Council & authorities re. location of services. Qualified operator. | 2 | 3 | 5 | y | Adopt both combined with cleared work area - essential personnel only. | CEO tm | Lift -2 week |
| 4 | CRANE Unsafe footing due to heavy rain & soft ground. | Crane may be damaged or topple causing injury to crew or bystanders. | 4 | 4 | 8 | Qualified operator. Consider weather and cancel or re-locate lift if in doubt. | 2 | 3 | 5 | n | Cancel or relocation preferred. | CEO tm RM | Lift -2 days |
| 5 | CRANE Overhead obstructions. | Crane may hit power lines, cables, poles or buildings causing injury to crew or workers. | 3 | 5 | 8 | Qualified operator. OTHR provide an independent spotter. | 2 | 2 | 4 | y | Adopt both. Arrange communication bet. Spotter & operator. | CEO tm RM | Lift day |
| 6 | CRANE Inappropriate lift crane, Incorrect or uncertified lifting gear. | Dropped load, Damage to railmotor. Injury to workers. | 4 | 4 | 8 | Prior approval of equipment and load certification All slings to be rated; weight of lift to be established and verified | 1 | 3 | 4 | y | Adopt combined with cleared work area - essential personnel only, and #3 below. | CEO tm RM | Lift -1 week |
| 7 | CRANE Incorrect spreader bars. | Dropped load, Damage to railmotor. Injury to workers. | 4 | 4 | 8 | Ensure correct spreaders by checking with rail mover & crane op. | 1 | 3 | 4 | y | Combined with above. | CEO tm RM | Lift -1 week |
| 8 | IN TRANSIT Removal from ACT to Oberon - under control of Mario Mencigar | Loss or damage to load. Accident in transit. Over width dangers. | 3 | 4 | 7 | Independently checked by OTHR prior to departure Ensure appropriate RTA/police and adequate insurance in place. | 2 | 4 | 6 | ? | Out of our control but carry out sufficient checks to reduce our liability. | CEO SEC | ASAP |
| 9 | UNLOADING Personnel encroach upon safety zone. | Injury to public, OTHR workers and unloading crew. | 2 | 5 | 7 | Briefing prior to start. Security barriers. Crowd control personnel. Evacuation point marked. | 1 | 2 | 3 | y | Combination of all controls. | CEO tm RM | Lift day |
| 10 | UNLOADING Unauthorised entry to safety zone. | Injury to intruder. | 2 | 5 | 7 | Police supervision. | 2 | 3 | 5 | n | Minimise by requesting Police supervision in advance. | SEC | Lift -2 weeks |
| 11 | UNLOADING Unloading crew hit by load. | Injury to crew. | 3 | 5 | 8 | Briefing prior to start. Clear hazards from work area. Wear PPE including hard hat. | 2 | 2 | 4 | y | Adopt all controls. | CEO tm RM | Lift day |
| 12 | UNLOADING Holes, Uneven surfaces Tripping hazards. | Trips, falls and injury | 4 | 3 | 7 | Fill holes. Level uneven surfaces. Remove all obstacles. | 2 | 2 | 4 | y | Adopt all controls. | tm atm | Lift -1 week |
| 13 | UNLOADING Load falls | Injury or death to crew. | 3 | 5 | 8 | No personnel to be under the load at any time. | 1 | 1 | 2 | y | Adopt. Supervise unloading team. | tm RM | Lift day |
| 14 | UNLOADING Locating onto rails. | Injury to crew. Pinch points. | 3 | 3 | 6 | Briefing prior to start. Wear PPE including gloves and hard hat. | 1 | 1 | 2 | y | Adopt all controls. | tm RM | Lift day |
| 15 | MOVE TO STORAGE Track fails. | Damage to railmotor. Injury to workers. Damage to property. | 3 | 5 | 8 | Inspect track before operation. | 1 | 1 | 2 | y | Adopt. | tm | Lift day |
| 16 | MOVE TO STORAGE Runaway due to rail not being level. | Damage to railmotor. Injury to workers. Damage to property. | 4 | 5 | 9 | Check gradient diagram. Check track level. Check CPH hand brake. Lock or chock wheels. Chain r/stock to tractor or excavator. | 1 | 1 | 2 | y | Adopt all controls. Check gradients before the day. Check handbrake while secured to transporter. | tm RM | Lift day |
| 17 | MOVE TO STORAGE Controlled movement to storage position results in runaway. | Damage to railmotor. Injury to workers including crushing during movement. Damage to property. | 2 | 4 | 7 | Brief machine operator towing rolling stock. Clear machine path. Use a spotter to advise operator and brake person. Check chains - use a back up chain or heavy rope. Control movement using r/stock handbrake. Chock track at intermediate locations. Tow at dead slow. Non essential crew to keep well clear. | 2 | 1 | 3 | y | Plan in advance. Adopt all controls. | CEO tm RM | Lift -1 week & Lift day |
| 18 | R/STOCK STORED Runaway due to missing chocks or restraints. | Damage to railmotor. Injury to workers including crushing during movement. Damage to property. | 4 | 5 | 9 | Fit effective restraints bolted to the line with backup sleepers on the line. Set & lock catch points and derailers between r/stock and Albion Street. Apply wagon handbrake & repair if ineffective. | 1 | 1 | 2 | y | Adopt all controls. Gradient is 1:174 towards Albion St. | ceo RM | Lift day |

ACTION BY KEY: CEO= President, OM= Operations Manager, SM= Systems Manager, SEC= Secretary, PO= Public Officer, TM= Track Manager includes ATM= Assistant TM, FC= Finance Co-ordinator, TR= Treasurer YET TO BE APPOINTED: IM= Infrastructure Manager, IA= Internal Auditor, RM=Rollingstock Manager

SIGNATURE SHEET - Risk Assessment for: ………………………………………………………………….……. (brief description)
Risk Assessment associated with OTHR operation described above.                          Date of Operation: …… /……. /2…….

| Risk No. | Hazard What can go wrong? | Risk What could happen? | Inherent Risk |  |  | Controls How could it be fixed? | Residual Risk |  |  | ALARP | Finding: Issues? Problems? Adopt? | Action |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Lik. | Cons | Risk |  | Lik. | Cons | Risk | ? |  | By | When |
| ALL | As described above. | I have discussed and understand the risks. |  |  |  | I will implement the Controls described above to the best of my ability so that the Risk is managed so that it is AS LOW AS REASONABLY PRACTICAL. |  |  |  |  | Note any issues you have with the Risks, Management or Controls. |  |  |
| OPERATING CREW. The operating crew on the day are requested to sign this document indicating that they have READ and UNDERSTOOD the RISKS and CONTROLS associated with this operation. |  | NAME (Print) ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… |  |  |  | POSITION (Task) ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… | signature ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… |  |  |  | COMMENT ………………………………………. ………………………………………. ………………………………………. ………………………………………. ………………………………………. ………………………………………. ………………………………………. ………………………………………. ………………………………………. ………………………………………. ………………………………………. ………………………………………. |  |  |