ACTION BY KEY: CEO= President, OM= Operations Manager, SM= Systems Manager, SEC= Secretary, PO= Public Officer, TM= Track Manager includes ATM= Assistant TM, FC= Finance Co-ordinator, TR= Treasurer, IM= Infrastructure Manager, IA= Internal Auditor, RM=Rollingstock Manager

Risks in OTHR operation: Unloading 4 Wheel Wagons   	Affects objective Safely unload wagons to RAIL & move to temporary storage area. 
Document Date: 02/2/2009. Reviewed: 08/09/12	Drafted by: SM          Assessed by: …………………..………

| Risk No. | Task & Hazard What can cause Harm? | Risk What could happen? (Triggers)” | Inherent Risk |  |  | Controls How could it be fixed? | Residual Risk |  |  | ALARP | Finding: Issues? Problems? Adopt? | Action |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Lik. | Cons | Risk |  | Lik. | Cons | Risk | ? |  | By | When |
| 1 | IN TRANSIT - Tilt Tray Removal from Orange to Oberon - under control of contractor using a tilt tray vehicle. | Loss or damage to load. Accident in transit. Over-size dangers. | 3 | 5 | 8 | Independently checked by OTHR prior to departure Ensure appropriate permits and adequate insurance in place. | 3 | 5 | 8 | ? | Out of our control but carry out sufficient checks to reduce our liability. | CEO SEC | ASAP |
| 2 | UNLOADING Personnel encroach upon safety zone. | Injury to public, OTHR workers and unloading crew. | 2 | 5 | 7 | Briefing prior to start. Security barriers & crowd control if required. Evacuation point marked. | 1 | 2 | 3 | y | Combination of all controls. | CEO /tm /RM | Lift day |
| 3 | UNLOADING Unauthorised entry to safety zone. | Injury to intruder. | 2 | 5 | 7 | Safety zone is within locked yard area. | 1 | 3 | 4 | y | Adopt & ensure gates are closed where possible. | CEO /tm /RM | Lift day |
| 4 | UNLOADING Unloading crew hit by load. | Injury to crew. | 3 | 5 | 8 | Briefing prior to start. Clear hazards from work area. All personnel to remain at the side of the vehicle and never behind the load. Wear PPE. | 2 | 2 | 4 | y | Adopt all controls. | CEO /tm /RM | Lift day |
| 5 | UNLOADING Holes, Uneven surfaces Tripping hazards. | Trips, falls and injury | 4 | 3 | 7 | Fill holes. Level uneven surfaces. Remove all obstacles. | 2 | 2 | 4 | y | Adopt all controls. | CEO /tm /RM | Lift -1 week |
| 6 | UNLOADING Winch fails and wagon rolls off the truck. | Injury or death to crew. Damage to load or property. | 3 | 5 | 8 | No personnel to be behind the vehicle and load. Chains and dogs to be progressively removed. Winch to be checked for ability to hold load before final chain removal. | 2 | 4 | 6 | y | Adopt. Supervise unloading team. | CEO /tm /RM | Lift day |
| 7 | UNLOADING Locating onto rails. | Load may topple sideways. Wheels may not align with rails. Injury or death to crew. Pinch points. | 3 | 5 | 8 | Briefing prior to start. Ensure unloading crew is standing sufficiently to one side to avoid crushing. Engage wheel flanges by careful and slow transfer to track. Wear PPE including gloves and hard hat. | 1 | 3 | 4 | y | Adopt all controls. Relocate tilt tray to correctly align with track. | CEO /tm /RM | Lift day |
| 8 15 | MOVE TO STORAGE Track fails. | Damage to wagon. Injury to workers. Damage to property. | 2 | 5 | 7 | Inspect track before operation. | 1 | 1 | 2 | y | Adopt. | CEO /tm /RM | Lift day |
| 9 | MOVE TO STORAGE Runaway due to rail not being level. | Damage to wagon. Injury to workers. Damage to property. | 4 | 5 | 9 | Check gradient diagram. Check track level. Check CPH hand brake. Lock or chock wheels. Chain r/stock to tractor or excavator. | 1 | 1 | 2 | y | Adopt all controls. Check gradients before the day. Check handbrake while secured to tilt tray - rectify if possible. | CEO /tm /RM | Lift day |
| 10 | MOVE TO STORAGE Controlled movement to storage position results in runaway. | Damage to wagon. Injury to workers including crushing during movement. Damage to property. | 2 | 4 | 6 | Brief machine operator towing rolling stock. Clear machine path. Use a spotter to advise operator and brake person. Check chains - use a back up chain or heavy rope. Control movement using r/stock handbrake if effective. Chock track at intermediate locations. Tow at dead slow. Non essential crew to keep well clear. | 2 | 1 | 3 | y | Plan in advance. Adopt all controls. | CEO /tm /RM | Lift -1 week & Lift day |
| 11 | R/STOCK STORED Runaway due to missing chocks or restraints. | Damage to wagon. Injury to workers including crushing during movement. Damage to property. | 4 | 5 | 9 | Fit effective restraints bolted to the line with backup sleepers on the line. Set & lock catch points and derailers between r/stock and Albion Street. Apply wagon handbrake & repair if ineffective. | 1 | 1 | 2 | y | Adopt all controls. Gradient is 1:174 towards Albion St. | ceo RM | Lift day |

ACTION BY KEY: CEO= President, OM= Operations Manager, SM= Systems Manager, SEC= Secretary, PO= Public Officer, TM= Track Manager includes ATM= Assistant TM, FC= Finance Co-ordinator, TR= Treasurer, IM= Infrastructure Manager, IA= Internal Auditor, RM=Rollingstock Manager

SIGNATURE SHEET - Risk Assessment for: ………………………………………………………………….……. (brief description)
Risk Assessment associated with OTHR operation described above.                          Date of Operation: …… /……. /2…….

| Risk No. | Hazard What can go wrong? | Risk What could happen? | Inherent Risk |  |  | Controls How could it be fixed? | Residual Risk |  |  | ALARP | Finding: Issues? Problems? Adopt? | Action |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Lik. | Cons | Risk |  | Lik. | Cons | Risk | ? |  | By | When |
| ALL | As described above. | I have discussed and understand the risks. |  |  |  | I will implement the Controls described above to the best of my ability so that the Risk is managed so that it is AS LOW AS REASONABLY PRACTICAL. |  |  |  |  | Note any issues you have with the Risks, Management or Controls. |  |  |
| OPERATING CREW. The operating crew on the day are requested to sign this document indicating that they have READ and UNDERSTOOD the RISKS and CONTROLS associated with this operation. |  | NAME (Print) ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… |  |  |  | POSITION (Task) ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… | signature ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… |  |  |  | COMMENT ………………………………………. ………………………………………. ………………………………………. ………………………………………. ………………………………………. ………………………………………. ………………………………………. ………………………………………. ………………………………………. ………………………………………. ………………………………………. ………………………………………. |  |  |