ACTION BY KEY: PR= President, OM= Operations Manager, SM= Systems Manager, SEC= Secretary, TM= Track Manager, IM= Infrastructure Manager, RM=Rolling stock Manager, TL=Team Leader, DR= Driver, OB= Observer

Risks in OTHR operation: Use of Sleeper Trolley     Affects objective: Safety issues during Sleeper Trolley operation
Document Date: 06/06/12 	Drafted by:  Ian Davis         Assessed by: PR/TM……………..………

REFERENCE:  SOP – 003 SLEEPER TROLLEY 060612

| Risk No. Hazard What can go wrong?; Risk What could happen?; Inherent Risk; Controls How could it be fixed?; Residual Risk; Finding: Issues? Problems? Adopt?; Action. L’hd.; Cons; Risk; L’hd.; PEC; Risk; By; When. |  |
| --- | --- |
|  |  | LOADING the SLEEPER TROLLEY |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Trolley could tip over when loaded |  | Injury to persons. Damage to trolley and/or load | 3 | 5 | 8 | Ensure all loaded material is centralised and secured. Place gluts over axles so load evenly displaced. | 2 | 2 | 4 | Adopt. Refer: SOP – 003 Sleeper Trolley | tm TL | During operation |  |
| 2 | Load interferes with braking operation |  | Trolley brakes cant be operated Collision with other vehicle Injury to persons | 2 | 5 | 7 | Sleepers to be loaded along the direction of rails to not interfere with brake operation | 2 | 2 | 4 | Adopt Refer: SOP – 003 Sleeper Trolley | tm TL | During operation |  |
| 3 | Trolley overloaded |  | Trolley could collapse injure persons and damage load. Brakes or wheel chocks may not hold overloaded trolley . Trolley could run away and injure persons or damage other infrastructure | 2 | 6 | 8 | Check load does not exceed the 1.5 tonne manufacturers specification. | 2 | 2 | 4 | Adopt Refer: SOP – 003 Sleeper Trolley | TL TM | During operation |  |
| 4 | Person injured |  | Person could be trapped by “pinch points” that occur during operation | 2 | 5 | 7 | All persons must be advised prior to loading of the possibility of the creation of “pinch points”during the operation. This should be done via a Toolbox Meeting prior to operation | 2 | 2 | 4 | Adopt Refer: SOP – 003 Sleeper Trolley | TL TM | Prior and during operation |  |

| Risk No. Hazard What can go wrong?; Risk What could happen?; Inherent Risk; Controls How could it be fixed?; Residual Risk; Finding: Issues? Problems? Adopt?; Action. L’hd.; Cons; Risk; L’hd.; PEC; Risk; By; When. |  |
| --- | --- |
|  | Using the Sleeper Trolley |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | Brakes fail | Trolley cant be stopped Injury to persons and/or infrastructure | 2 | 4 | 6 | Brakes to be checked prior to operation. Trolley only to be used at a slow walking pace | 2 | 2 | 4 | Adopt Refer: SOP – 003 Sleeper Trolley | tm TL | Prior and during operation |  |
| 5 | Trolley runs away | Trolley cant be stopped Injury to persons and/or infrastructure | 2 | 3 | 5 | When stopped the trolley brakes must be applied and the wheels chocked | 2 | 2 | 4 | Adopt Refer: SOP – 003 Sleeper Trolley | tm TL | During operation |  |
| 6 | Trolley connects with person | Injury to person | 2 | 4 | 6 | All persons to wear appropriate PPR so they are visible TL controls movement of trolley | 2 | 2 | 4 | Refer: SOP – 003 Sleeper Trolley | tm TL | During operation |  |
| 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |

ACTION BY KEY: PR= President, OM= Operations Manager, SM= Systems Manager, TM= Track Manager, IM= Infrastructure Manager , RM=Rollingstock Manager, DR = Driver , OB = Observer

SIGNATURE SHEET - Risk Assessment for: …Sleeper Trolley Operation     …
Risk Assessment associated with OTHR operation described above.                          Date of Operation:

| Risk No. | Hazard What can go wrong? | Risk What could happen? | Inherent Risk |  |  | Controls How could it be fixed? | Residual Risk |  |  | Finding: Issues? Problems? Adopt? |  | Action |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | L’hd. | Cons | Risk |  | L’hd. | Cons | Risk |  |  | By | When |  |
| ALL | As described above. | I have discussed and understand the risks. |  |  |  | I will implement the Controls described above to the best of my ability so that the Risk is managed so that it is AS LOW AS REASONABLY PRACTICAL. |  |  |  |  | Note any issues you have with the Risks, Management or Controls. |  |  |  |
| OPERATING CREW. The operating crew on the day are requested to sign this document indicating that they have READ and UNDERSTOOD the RISKS and CONTROLS associated with this operation. |  | NAME (Print) ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… |  |  |  | POSITION (Task) ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… | signature ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… |  |  |  | COMMENT ………………………………………. ………………………………………. ………………………………………. ………………………………………. ………………………………………. ………………………………………. ………………………………………. ………………………………………. ………………………………………. ………………………………………. ………………………………………. ………………………………………. |  |  |  |