CONTEXT & BACKGROUND

#### Introduction

The Oberon Tarana Railway (OTHR) needs to acquire five sets of points and 500 metres of 60lb railway track to complete the tracks into their newly completed railway carriage shed. To obtain these OTHR sent requests to State Rail and was granted permission to take the required from the railway yards at Coolah and the siding at Leadville.

OTHR have visited both these sites and have arranged for a team of fettlers to recover the required items in July 2013

#### Recovery Operations

This risk assessment is specifically for the removal of the required sets of points and the rail needing to complete the Oberon yard. The required items will be prepared for shipment and will be transported to Oberon by road.

The road transport will be facilitated by using contractors to provide the crane for loading and suitable vehicles for transport.

#### Safeworking

Track work will be carried out in accordance to safeworking practices that apply to any track maintenance work that is currently being done on the Oberon-Tarana line.

#### Personnel

All personnel associated with the track work are instructed to keep well clear of any mechanised equipment and must at all time wear appropriate PPE including hardhat if working in the vicinity of machinery.

As far as is reasonably practicable, the number of personnel on the site is kept to the minimum needed for operations.

#### Public/spectator risks

The removal of the required track and points should not attract public interest so no problems are anticipated.

RISK ASSESSMENT for RECOVERY of POINTS and TRACK from COOLAH and LEADVILLE

Date of Operation:	From July 2013

Operating Scenario:	Recovery of track and points from Coolah and Leadville

| Risk | Identified Event / Risk | Event/Risk Assessment |  | Analysis |  | Resid. Risk | Treatment/Control | Control |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref. |  | Likelihood | Consequences | Likelihood | Consequences | Rating |  | Responsibility |
| ATM/L1 | Environmental hazards | Ref: RM 101, TB-005 TB-007 TB-011 |  |  |  |  |  |  |
| ATM/L1.1 | Holes Uneven surfaces - Tripping hazards | Likely | Minor | F3 | C2 | D | ▪ Make all workers aware of holes. ▪ Advise uneven surfaces. ▪ Pre work briefing. | Ops Mgr, TM ATM, crew |
| ATM/L1.2 | - Barbed wire fencing causing snag injuries | likely | Minor | F3 | C2 | D | Barbed wire is an injury danger to the volunteers. Advise workers of danger and remove any loose wire before work commences | Ops Mgr, TM ATM, crew |
| ATM/L1.3 | Trip hazards - Rail lines - undergrowth | Likely | Minor | F3 | C2 | D | Pre work briefing PPE must be worn Re TB-002 | Ops Mgr, TM ATM, crew |
| ATM/L1.4 | - fire hazard. | Unlikely | Major | F3 | C2 | D | Eliminate dangerous vegetation growth. ▪ Minimise the accumulation of other combustible materials When using power tools be aware of spark hazards Have a fire extinguisher available | Ops Mgr, TM ATM, crew |

| ATM/L2 | Machinery Operations | Ref: TB-016 TB-006 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ATM/L2.1 | - Working near or with machinery - Collision with workers - Injury to workers | Unlikely | Major | F5 | C3 | C+ | All workers to wear correct PPE - High vis wear - Hardhats - Steel capped boots - Eye and ear protection - Gloves All workers to be made aware of dangers by pre-work briefings Re TB-002 | Ops Mgr, crew, ATM TM |
| ATM/L2.2 | - Machine operators to be licensed and experienced | Likely | Minor | F5 | C1 | E | Qualification checks must be made prior to operation | Ops Mgr, TM ATM, crew |
| ATM/L3 | Personnel | Ref:Pol-003 POL-004 TB-007 TB-011 |  |  |  |  |  |  |
| ATM/L3.1 | Volunteer error or ill health - due to staff error, due to intoxication, ill health, fatigue | unlikely | Major | F5 | C2 | C+ | OTHR D & A program OTHR health assess’t OTHR Fatigue mgt program | Ops Mgr, crew TM, ATM |
| ATM/L4 | Trips, slips & falls | Likely | Minor | F5 | C2 | C+ | Worksite risk assessments & controls Corridor inspection and maintenance | Ops Mgr, TM ATM, crew |

Risk Review Version	1

Date of Risk Review:	24 June 2013

Compiled by:	Ian Davis (Assistant Track Manager OTHR)

RISK ASSESSMENT METHODOLOGY

Risk Analysis Matrix

The following Risk Analysis Matrix shows the approach and the description of levels used in a safety context by the NSW Rail Transport Museum.

|  |  | Likelihood |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Less than once every 10,000 years | Less than once every 1,000 years | Once every 100 to 1,000 years | Once every 10 to 100 years | Once every 1 to 10 years | More than once per year |
| Risk assessment criteria for the NSW Rail Transport Museum |  | Theoretically possible but not heard of in the rail industry | Has happened rarely in rail industry, not in the RTM. Very unlikely in the RTM | Heard of in heritage operations, not in the RTM. Unlikely in the RTM | Occasionally heard of in heritage operations. Happens annually in wider rail industry | Likely to occur in the RTM once per year or every few years | Likely to occur frequently in the RTM – several times per year |
| Consequence |  | F1 | F2 | F3 | F4 | F5 | F6 |
| Catastrophic - multiple fatalities, damage in excess of $10 million | C6 | C+ | B- | B+ | A | A | A |
| Disastrous - Single fatality, several serious injuries, damage in excess of $1 million | C5 | C- | C+ | B- | B+ | A | A |
| Critical - Serious injury or damage, long term impacts, costs in excess of $50,000 | C4 | D | C- | C+ | B- | B+ | A |
| Major - hospitalisation, lost time injury, costs incurred between $1,000 and $50,000 | C3 | D | D | C- | C+ | B- | B+ |
| Minor - First Aid treatment required, no LTI, costs incurred less than $1,000 | C2 | D | D | D | C- | C+ | B- |
| Negligible - Record in injury register, cost to RTM is insignificant | C1 | E | E | E | E | E | E |

|  | Unacceptable |  | ALARP range |  | Acceptable |  | Low |
| --- | --- | --- | --- | --- | --- | --- | --- |

Risk Criteria

The letters in the matrix (Risk Rating) are an indication of the level and hence acceptability of risk, enabling action and controls to be prioritised on the basis of cost-effectiveness and reduction of risk to ‘ALARP’ (As Low As Reasonably Practicable) levels, and can be interpreted as follows:

| Risk Rating | Level | Descriptor – Risk Criterion | Priority Actions |
| --- | --- | --- | --- |
| A | Intolerable | Unacceptable – risk cannot be justified save in extraordinary circumstances | Stop action immediately, pending implementation of additional controls to reduce risk. |
| B+ | Very high | Unacceptable; risk may be able to be justified in exceptional circumstances; additional controls needed urgently | Urgently seek additional controls; if not practicable, consider stopping activity. Safety & Environment Review Committee attention. |
| B- | High (ALARP) | Tolerable if ALARP – look for additional controls to drive towards broadly acceptable | Seek additional controls if practicable. General Manager attention. Ongoing review. |
| C+ | Significant (ALARP) | Tolerable if ALARP – look for additional controls to drive towards broadly acceptable | Seek additional controls if practicable. General Manager attention. Ongoing review. |
| C- | Significant (ALARP) | Acceptable if ALARP – ideally look for additional controls | General Manager attention. Ongoing review. |
| D | Moderate | Broadly acceptable | Ongoing monitoring of procedures and controls. |
| E | Low | Acceptable | Maintain existing controls & procedures. |

In addition, all high consequence (C5 or C6) risks should be reviewed and additional controls considered to reduce likelihood, even if assessed and ranked as ALARP.  So far as is reasonably practicable, those hazards representing the greatest risk should be given priority.