ACTION BY KEY: PR= President, OM= Operations Manager, SM= Systems Manager, SEC= Secretary, PO= Public Officer, TM= Track Manager, FM= Finance Manager, TR= Treasurer, IM= Infrastructure Manager, IA= Internal Auditor, RM=Rollingstock Manager, TL=Team Leader

Risks in OTHR operation: Oberon Yard Wagon Unload to Rail     Affects objective: Safely unload wagon, from transport to RAIL & move to temporary storage area.
Document Date: 27/09/2010 	Drafted by:  SM         Assessed by: Ian Davis……..………

| Risk No. | Hazard What can go wrong? | Risk What could happen? | Inherent Risk |  |  | Controls How could it be fixed? | Residual Risk |  |  | ALARP | Finding: Issues? Problems? Adopt? | Action |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | L’hd. | Cons | Risk |  | L’hd. | PEC | Risk | ? |  | By | When |
|  | TRANSPORT WAGON TO OBERON |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Transport Vehicle Unqualified and/or uninsured operator. | Injury or death to personnel and property damage. | 3 | 5 | 10 | * Independent check by OTHR on qualifications, currency and insurance. | 2 | 1 | 3 | y | Adopt. File copies with DC. | PR SEC dc | Arrive -1 week |
| 2 | Transport Vehicle Safely transport wagon from pick-up point to Oberon | Transport breakdown or miss-hap enroute | 5 | 3 | 8 | Use of specialised rail transporter | 2 | 1 | 3 | y | Adopt. This operator is well known to OTHR teams and the PR. | PR tm RM | Arrive -1 day |
| 3 | Transport Vehicle Unsafe footing due to underground services, trenches or embankments. | Transport vehicle has negotiated unloading route twice in the past without incident | 4 | 5 | 9 | * Check with Council & authorities re. location of services. * Qualified operator. | 2 | 3 | 5 | y | Adopt. Services have been previously located. | PR tm | Arrive -1 day |
| 4 | Transport Vehicle Unable to reverse up ground in front of station | Vehicle could become bogged in soft ground | 4 | 6 | 6 | Vehicle will be directed up embankment that was previous road and is solid | 2 | 1 | 2 | Y | Adopt Remove section of fence to facilitate | PR TM | Arrive -2 hrs |
| 5 | Transport Vehicle Unable to line up transport with rail | Wagon wont be able to be unloaded | 3 | 5 | 6 | Operator and assistant will guide transport into correct position. TM will ensure track is accessable | 2 | 1 | 3 | Y | Adopt | PR TM | Arrive |
| 6 | Preparation for next stage Wagon may roll away | Wagon may roll away when unloaded from transport | 4 | 5 | 9 | * Apply hand brake. * Have wheel chocks ready. * Clamp rail stop to down gradient side. * Use a sleeper across the line if in doubt. | 1 | 3 | 4 | Y | Adopt all controls - additional sleepers could be used between items of other rolling stock if necessary | PR TM | Arrive |

|  | LOCATING WAGON TO RAIL |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | Aligning Transport to Rail Transport cant align with track | Unable to unload wagon using the transports inbuilt mechanism | 2 | 3 | 6 | Transport operation crew to perform this task. OTHR to have contingency plan is wagon cant be unloaded in the normal manner | 2 | 2 | 4 | y | Adopt. Have contingency plan of crane lift if transport cant be unloaded. | PR tm RM | Arrive day |
| 8 | Unloading to Rail | Wagon runaway | 3 | 3 | 6 | Implement step 6 to ensure wagon wont roll on rail | 1 | 1 | 2 | y | Adopt all controls. | PR tm RM | Arrive day |
|  | MOVING WAGON TO SECURE STORAGE POSITION |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | Move to Storage Track fails. | Damage to wagon. Injury to workers. Damage to property. | 3 | 5 | 8 | * Inspect track before operation. | 1 | 1 | 2 | y | Adopt. | tm | Arrive day |
| 17 | Move to Storage Runaway due to rail not being level. | Damage to wagon. Injury to workers. Damage to property. | 4 | 5 | 9 | * Apply wagon handbrake & repair if ineffective. * Fit effective backup restraints -sleepers bolted across the line with backup sleepers on the line. | 1 | 1 | 2 | y | Adopt all controls. Gradient is 1:174 towards Albion St. | PR tm RM | Arrive day |
| 18 | Move to Storage Controlled movement to storage position results in runaway. | Damage to wagon, other rollingstock or infrastructure. Injury to workers including crushing during movement. Damage to property. | 2 | 4 | 6 | * Brief machine operator towing rolling stock. * Clear machine path. * Use a spotter to advise operator and brake person. * Check chains and shackles. * Control movement using r/stock handbrake if effective. * Chock track at intermediate locations. * Tow at dead slow. * Non essential crew to keep well clear. | 2 | 1 | 3 | y | Plan in advance. Equipment used: 15 tonne tractor crane straddling track, shackled to wagon with a rigid “stiff bar”. Adopt all controls. | PR tm RM | Arrive day |
|  | SECURE STORAGE |  |  |  |  |  |  |  |  |  |  |  |  |
|  | The following plan indicates where it is proposed to relocate the 4 wheel wagons onto the line. The red box is the new wagon. |  |  |  |  |  |  |  |  |  |  |  |  |

ACTION BY KEY: PR= President, OM= Operations Manager, SM= Systems Manager, SEC= Secretary, PO= Public Officer, TM= Track Manager includes ATM= Assistant TM, FC= Finance Co-ordinator, TR= Treasurer YET TO BE APPOINTED: IM= Infrastructure Manager, IA= Internal Auditor, RM=Rollingstock Manager

SIGNATURE SHEET - Risk Assessment for: Transport and unloading of Wagon

Risk Assessment associated with OTHR operation described above.                          Date of Operation: 28/10 /2010

| Risk No. | Hazard What can go wrong? | Risk What could happen? | Inherent Risk |  |  | Controls How could it be fixed? | Residual Risk |  |  | ALARP | Finding: Issues? Problems? Adopt? | Action |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | L’hd. | Cons | Risk |  | L’hd. | Cons | Risk | ? |  | By | When |
| ALL | As described above. | I have discussed and understand the risks. |  |  |  | I will implement the Controls described above to the best of my ability so that the Risk is managed so that it is AS LOW AS REASONABLY PRACTICAL. |  |  |  |  | Note any issues you have with the Risks, Management or Controls. |  |  |
| OPERATING CREW. The operating crew on the day are requested to sign this document indicating that they have READ and UNDERSTOOD the RISKS and CONTROLS associated with this operation. |  | NAME (Print) ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… |  |  |  | POSITION (Task) ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… | signature ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… |  |  |  | COMMENT ………………………………………. ………………………………………. ………………………………………. ………………………………………. ………………………………………. ………………………………………. ………………………………………. ………………………………………. ………………………………………. ………………………………………. ………………………………………. ………………………………………. |  |  |