ACTION BY KEY: PR= President, OM= Operations Manager, SM= Systems Manager, SEC= Secretary, TM= Track Manager, IM= Infrastructure Manager, RM=Rolling stock Manager, TL=Team Leader, DR= Driver, OB= Observer

Risks in OTHR operation: Train movement within Oberon Yard     Affects objective: Safety issues during train and wagon movement within Oberon Yard.
Document Date: 15/4/11 	Drafted by:  Ian Davis         Assessed by: …………………..………

| Risk No. | Hazard What can go wrong? | Risk What could happen? | Inherent Risk |  |  | Controls How could it be fixed? | Residual Risk |  |  | Finding: Issues? Problems? Adopt? | Action |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | L’hd. | Cons | Risk |  | L’hd. | PEC | Risk |  | By | When |  |
|  | The movement of the two 73 class diesels, 7307 and 7321, can be broken down into small increments each with controls put into place to minimise any risk to personnel or infrastructure. There are a number of track considerations that have to be understood before we begin the train movement. Refer to the attached diagram for the path of the move and the infrastructure points on the route. Track Considerations |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Point “G” aligned to Run-around loop and clipped | Engine would take wrong path within station precinct. | 2 | 2 | 2 | Ensure Shunter has checked and clipped points | 2 | 6 | 2 | Adopt. | SM tm TL | Prior to operation |  |
| 2 | Point “F” aligned to Run-around loop and clipped with an SL clip | Engine would force its way through points which could damage rail | 2 | 5 | 3 | Ensure Shunter has checked and clipped points Observer to check with Shunter prior to train movement | 2 | 6 | 3 | Adopt | SM tm TL | Prior to operation |  |
| 3 | Engines current positions are on coal siding 9metres from point “G”. | No risk positional information only | 2 | 2 | 2 | Movement crew to be fully briefed and supervised | 2 | 8 | 4 | Adopt. | tm TL | During operation |  |

| Risk No. Hazard What can go wrong?; Risk What could happen?; Inherent Risk; Controls How could it be fixed?; Residual Risk; Finding: Issues? Problems? Adopt?; Action. L’hd.; Cons; Risk; L’hd.; PEC; Risk; By; When. |  |
| --- | --- |
|  | Engine movement The engine crew will comprise a qualified driver, familiar with 73 class diesel, and an observer. There will also be a Shunter on the ground to control the positioning of the points and to be in constant contac with the Observer |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Ensure track is clear | Remove all trolleys, tools, obstructions within 4ft of track and any persons that are not part of movement team. | 2 | 6 | 5 | Check track for any obstruction. Remove persons not on movement team. Shut all access to station precinct. | 2 | 5 | 5 | Adopt | SM tm TL | Prior to operation |  |
| 2 | Verify engines are operational | Check running of engines Check brakes Check safety equipment (horn etc) . | 2 | 6 | 5 | Run engines prior to moving day to check operation. On morning of move re-check | 3 | 5 | 5 | Adopt | tm RM TL | Prior to operation |  |
| 3 | Move engine 7307through points “G” and “F”- 130metres | Engine could be diverted to wrong line because point set incorrectly | 2 | 3 | 2 | Observer in constant interaction with Shunter Engine limited to 5kph (walking speed) Movement team has been fully briefed prior to any operation. Team Leader observing operation from station. | 2 | 6 | 2 | Adopt | SM TM TL RM | During operation |  |
| 4 | When 7307 arrives at destination, secure loco. | Brakes wont hold loco | 2 | 5 | 3 | Observer remains on board Driver ensures brakes are locked on | 2 | 6 | 2 | Adopt | SM RM TL DR | During operation |  |
| 5 | Move engine 7321 through points “G” and “F”- 130metres, finishing behind 7307 | Engine could be diverted to wrong line because point set incorrectly | 2 | 3 | 2 | Observer in constant interaction with Shunter Engine limited to 5kph (walking speed) Movement team has been fully briefed prior to any operation. Team Leader observing operation from station | 2 | 6 | 2 | Adopt | SM TM TL RM | During operation |  |

| Risk No. Hazard What can go wrong?; Risk What could happen?; Inherent Risk; Controls How could it be fixed?; Residual Risk; Finding: Issues? Problems? Adopt?; Action. L’hd.; Cons; Risk; L’hd.; PEC; Risk; By; When. |  |
| --- | --- |
|  | Engine movement The engine crew will comprise a qualified driver, familiar with 73 class diesel, and an observer. There will also be a Shunter on the ground to control the positioning of the points and to be in constant contac with the Observer |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 | When 7321 arrives at destination, secure loco. | Brakes won’t hold loco | 2 | 5 | 3 | Observer remains on board Driver ensures brakes are locked on | 2 | 6 | 2 | Adopt | SM RM TL DR | During operation |  |
| 7 | Shunter removes SL clip from point “F”, reverses point “F” to lie to loop then reinstalls SL clip at point “F”. | Shunter fails to position points correctly SL lock not fitted properly | 2 | 5 | 3 | Observer watches operation from cabin Team leader observes from platform | 2 | 6 | 2 | Adopt | TM RM TL OB | During operation |  |
| 8 | Check point “E” is laying to loop | Loco would push through points at could damage line | 2 | 5 | 3 | Observer and Team leader checks that Shunter has positioned points correctly | 2 | 6 | 2 | Adopt | TM RM TL OB | During operation |  |
| 9 | Loco 7321 reversed along loop to clear point “E”- 150metres | Loco could runaway | 2 | 5 | 3 | Engine limited to 5kph (walking speed) Movement team has been fully briefed prior to any operation. Team Leader observing operation from station. | 2 | 6 | 2 | Adopt | TM RM TL OB | During operation |  |
| 10 | Shunter removes SL clip from point ”F” , walks to point “E” , reverses point “E” to lay to platform line and secures with SL clip. | Loco could be directed to wrong track if point laying to wrong track | 2 | 3 | 3 | Observer watches operation from cabin Team leader observes from platform | 2 | 6 | 2 | Adopt | TM RM TL OB | During operation |  |
| 11 | Loco 7321 moves on main line to designated stabling point, shut down and secured | Loco could runaway | 2 | 5 | 3 | Engine limited to 5kph (walking speed) Movement team has been fully briefed prior to any operation. Team Leader observing operation from station. | 2 | 6 | 2 | Adopt | TM RM TL OB | During operation |  |

| Risk No. Hazard What can go wrong?; Risk What could happen?; Inherent Risk; Controls How could it be fixed?; Residual Risk; Finding: Issues? Problems? Adopt?; Action. L’hd.; Cons; Risk; L’hd.; PEC; Risk; By; When. |  |
| --- | --- |
|  | Engine movement The engine crew will comprise a qualified driver, familiar with 73 class diesel, and an observer. There will also be a Shunter on the ground to control the positioning of the points and to be in constant contac with the Observer |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 | Shunter removes SL clip from point “E” and reverses point to lay to loop | Loco could be directed to wrong track if point laying to wrong track | 2 | 3 | 3 | Observer watches operation from cabin Team leader observes from platform | 2 | 6 | 2 | Adopt | TM RM TL OB | During operation |  |
| 13 | Shunter walks back to frame “F” , verifies point is laying to loop and secures with SL clip, | Loco could be directed to wrong track if point laying to wrong track | 2 | 3 | 3 | Observer watches operation from cabin Team leader observes from platform | 2 | 6 | 2 | Adopt | TM RM TL OB | During operation |  |
| 14 | Loco 7307 moves along loop to clear point “E”- 150metres | Loco could runaway | 2 | 5 | 3 | Engine limited to 5kph (walking speed) Movement team has been fully briefed prior to any operation. Team Leader observing operation from station. | 2 | 6 | 2 | Adopt | TM RM TL OB | During operation |  |
| 15 | Shunter removes SL clip from point ”F” , walks to point “E” , reverses point “E” to lay to platform line and secures with SL clip. | Loco could be directed to wrong track if point laying to wrong track | 2 | 3 | 3 | Observer watches operation from cabin Team leader observes from platform | 2 | 6 | 2 | Adopt | TM RM TL OB | During operation |  |
| 16 | Loco 7307 moves on main line to designated stabling point, shut down and secured | Loco could runaway | 2 | 5 | 3 | Engine limited to 5kph (walking speed) Movement team has been fully briefed prior to any operation. Team Leader observing operation from station. | 2 | 6 | 2 | Adopt | TM RM TL OB | During operation |  |
| 17 | Shunter returns all points to original positions and removes SL clip. |  |  |  |  |  |  |  |  |  |  |  |  |

ACTION BY KEY: PR= President, OM= Operations Manager, SM= Systems Manager, TM= Track Manager, IM= Infrastructure Manager , RM=Rollingstock Manager, DR = Driver , OB = Observer

SIGNATURE SHEET - Risk Assessment for: …Train movement within Oberon Yard     …
Risk Assessment associated with OTHR operation described above.                          Date of Operation: 30/4/11

| Risk No. | Hazard What can go wrong? | Risk What could happen? | Inherent Risk |  |  | Controls How could it be fixed? | Residual Risk |  |  | Finding: Issues? Problems? Adopt? |  | Action |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | L’hd. | Cons | Risk |  | L’hd. | Cons | Risk |  |  | By | When |  |
| ALL | As described above. | I have discussed and understand the risks. |  |  |  | I will implement the Controls described above to the best of my ability so that the Risk is managed so that it is AS LOW AS REASONABLY PRACTICAL. |  |  |  |  | Note any issues you have with the Risks, Management or Controls. |  |  |  |
| OPERATING CREW. The operating crew on the day are requested to sign this document indicating that they have READ and UNDERSTOOD the RISKS and CONTROLS associated with this operation. |  | NAME (Print) ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… ………………………………………………… |  |  |  | POSITION (Task) ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… | signature ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… ………………………… |  |  |  | COMMENT ………………………………………. ………………………………………. ………………………………………. ………………………………………. ………………………………………. ………………………………………. ………………………………………. ………………………………………. ………………………………………. ………………………………………. ………………………………………. ………………………………………. |  |  |  |

Station Diagram with infrastructure referred to in RM-123

Point “F”                                     Point “E”                                          Point “G”

Cattle loading line

7321

7307

Run-around line

Main Line

7321           7307   Final positions

Station