|  | PO Box 299 Oberon NSW 2787 ABN 98 107 506 208 | Version | Author | Reviewed | Reference |
| --- | --- | --- | --- | --- | --- |
|  |  | A - Rev 0 | SM | SEC/PR | SOP-004 |
|  |  | 8 May 2010 |  |  |  |
|  |  | Standard Operating Procedure |  |  |  |

SOP - HOT WORKING

Document Status

| Version | Date | Revision | Prepared | Reviewed | Approved |
| --- | --- | --- | --- | --- | --- |
| Draft | 8 May 2010 |  | SM (Fletcher) | Mgt. Cttee. |  |
| A | May 2010 | Rev.0 | SM (Fletcher) |  | Management Committee |

Revision Record

| Revision | Date Issued: | Description of change: |
| --- | --- | --- |
| 1 |  |  |

Distribution

| Hard Copy | Position Title | Electronic Controlled Copy |
| --- | --- | --- |
| A-0 | Management Committee | OTHR Website |

OPERATING CREWS NEED TO READ THIS SOP & SIGN BELOW

Signatures:  (attach F-011 Signature Sheet if required)

| ## Name (Print) | Signature | Name (Print) | Signature |
| --- | --- | --- | --- |

Objectives:

The primary objective of the OTHR Hot Work Procedure is to keep heat away from surrounding combustibles, so that IGNITION DOES NOT OCCUR.

Secondary objectives are-

- to ensure that the person(s) undertaking Hot Work can quickly put out any small fire should it occur.

- to ensure the occupational health and safety of personnel performing and surrounding the hot work

- to ensure that there is minimal disruption and property damage.

Definition:

Hot Work is any gas cutting, welding, grinding, use of naked flames or other activities which may produce sparks or high temperatures that can set things on fire.

Responsibilities

### Hot Worker

- Obtaining a signed hot work permit prior to commencement of hot work.

- Ensuring the conditions of the hot work permit are met and maintained for the duration of the hot work.

- Displaying the hot work permit on the job or hot work equipment (e.g. grinder, welder or oxy set, etc).

- Conducting an inspection of the hot work site 30 minutes after completion of the hot work to ensure the site remains or is made safe / fire free.

- Returning the hot work permit to the Document Office, attaching it to the Hot Work Register and signing off the job in the hot work register (R-013 Hot Work Permit Register)

- Notifying the permit issuer when hot work is completed.

Permit Issuer

- Issuing a permit only if they believe the hot work can be performed safely and without fire.

- Inspecting the site prior to approval of the hot work permit.

- Ensuring all conditions of the hot work permit, are fully understood.

- Ensuring the hot work is entered in the Hot Work Register immediately the permit is issued.

- Conducting checks of hot work conditions while hot work is in progress.

- Conducting or arranging for the inspection of the hot work site 1 to 2 hours after the completion of hot work.

Fire Watcher

- To watch everywhere that sparks may go

- To standby with fire extinguisher and/or hose ready to extinguish any fire

- To stop the hot worker immediately sparks go anywhere they cannot be seen

- To not be distracted by or involved in other work.

Document Controller

- The Document Controller shall arrange for the collection and filing of returned hot work permits.

## Advisory Information

Fire Watcher

No person may be appointed a fire watcher unless that person -

- Is of a reliable nature.

- Is or is known to have been instructed in the detailed use of any fire fighting equipment required for the hot work.

- Has been clearly instructed as to his/her responsibilities.

- Has been shown the location and method of operation of the nearest fire equipment.

- Has no other tasks during the hot work.

## Preparing for Hot Work

Remember the Hierarchy of Controls for Hot Work. If possible:

- Eliminate the hot work. Do it another way.

- Substitute a safer hot work method considering the characteristics of the hot work method, the job and the site.

- Isolate the job from the fuel or the fuel from the job. Aim for 10m clear.

- Engineering Controls – Establish barriers between the hot work and the fuel, welding drapes, wet down the fuel and floors, etc

- Administrative Controls – use fire watchers, permits and hot work procedure, etc

- Personal Protective Equipment. – wear appropriate PPE.

### Performing the Hot Work

- Fire watch commences immediately the hot work starts.

- The fire watcher(s) must be able to see everywhere that sparks may go.

- The fire watcher(s) must not have any other work to do other than to watch for fires.

- If the fire watcher needs to leave to job/site, he/she must stop the hot work until they return or are replaced.

- Team Leader checks that conditions are maintained during the job. Conditions may change with wind and weather or other work in the area.

### After Completion of Hot Work

- Hot Worker packs up equipment and does a check around the job site for smoulders or signs of fire.

- Hot Worker Returns the Hot Work Permit to the Document Office.

- Hot Worker Signs off the Hot Work Register

- Hot Worker Notifies the Permit Issuer that the work is completed

- Two hours after the Completion of Hot Work the Permit Issuer completes a check of the job site to look for signs of fire or smoulders.