| PO Box 299 Oberon NSW 2787 ABN 98 107 506 208 | Version | Author | Reviewed | Reference |
| --- | --- | --- | --- | --- |
|  | A-Rev2 | SMS Work party |  | SOP-006 |
|  | 23/04/2018 |  |  |  |
|  | Clipping Points – Yard Operation |  |  |  |

Document Status

| Version | Date | Revision | Prepared | Reviewed | Approved |
| --- | --- | --- | --- | --- | --- |
| A | 23/11/2012 |  | SM | Committee | 24/11/2012 |
| A-Rev2 | 01/04/2018 | 2 | SMS Working Party | Committee | 23/04/2018 |

Revision Record

| Revision | Date issued | Description of Changes |
| --- | --- | --- |
| 1 | 24/11/2012 | Minor changes |
| 2 | 01/04/2018 | Formatting, deleting Swing Nose and Tangential crossings |

Clipping Points – Yard Operation

SOP-006

1: Introduction

Point clips are used to secure points in the normal or reverse position.

Facing points on running lines must be locked if:

- you cannot be sure that the points will remain in the correct position, or

- there is an instruction at the points to clip and lock them.

A lock may also be used on the clip to ensure the point clip is not removed, authority to use a lock or not is with the Track Manager.

2: Using Point Clips

Use standard point clips for standard points. Standard points have a switch blade and stock rail of similar depth. The standard point clip has a simple hook for the switch blade flange.