|  | PO Box 299 Oberon NSW 2787 ABN 98 107 506 208 | Version | Author | Reviewed | Reference |
| --- | --- | --- | --- | --- | --- |
|  |  | A - Rev 1 | SM | SEC/TM | SOP-00X |
|  |  | 4th January 2011 |  |  |  |
|  |  | Loco Class 73 - Yard Operation |  |  |  |

STANDARD OPERATING PROCEDURE.

Safe Working Instruction – Loco Class 73 within Oberon Station Yard.

Any locomotive used within the area defined as Oberon yard will be operated subject to the conditions and definitions specified below.

- UNDER NO CIRCUMSTANCES CAN THE LOCOMOTIVE BE OPERATED BY AN UNAUTHORISED PERSON.

- The locomotive is only to be operated for the purpose of MAINTENANCE or testing.

- Fettlers working in the DANGER ZONE (see definition below) must have a Track Safety Awareness Certificate – check with your Team Leader.

- Operations will be carried out between SUNRISE and SUNSET only.

- Drivers and crew are bound by OTHR policies such as: Worker Safety; Drug and Alcohol use; Fatigue Management etc.

- Locomotives are not to be used for carrying passengers other than OTHR track workers or representatives of statutory authorities engaged in testing or accrediting the Locomotive.

- WRITTEN AUTHORITY to move the Locomotive must be obtained from the president and the operations manager (or in his/ her absence the track manager).

- The authority specified in “7.” above can only be issued to an AUTHORIZED LOCOMOTIVE DRIVER.

- This authority can only be issued if NO MOVEMENT OF OTHER ROLLING STOCK is at that time authorised or occurring within the station yard and if the Oberon down home signal (if installed and operational) is placed at danger.

- The written authority will be in the form as shown in Appendix “A” (AUTHORITY TO OPERATE SOP – 001A) attached to this instruction.

- Having received the completed “Authority to Operate Loco Class 73 in Oberon Yard” the authorised driver will carry out a DAILY LOCOMOTIVE CHECK SOP – 001B as shown in Appendix “B” to satisfy (in writing) to his/her self that the Loco is safe to operate.

- The authorised driver will use the Loco to complete the required task in a safe manner and in the shortest possible time.

- Maximum speed of the Loco within the Oberon station yard is 5KPH (walking speed).

- The Loco must operate at DEAD SLOW speed when near other rail safety workers, pedestrians, plant, machinery, danger zones or when reversing.

- All crew members must wear appropriate PPE including an approved safety vest.

- The Loco must carry a prescribed size of RED FLAG which is to be displayed when the car is stationary or stopping.

- The horn and safety beacon must be tested before operations commence and the safety beacon must be ON during all operations.

- Heavy equipment must be stowed in a manner which prevents injury to crew should the Loco stop suddenly or be involved in an accident.

- It is the responsibility of the authorized driver or designated crew member to set any points encountered to the correct setting prior to the Loco reaching the point (trailing through wrongly set points is not permitted).

- No passengers are to be carried at any time on the Loco or any towed carriage until full certification is granted

- The driver must refuse to move the Loco if other workers contravene these instructions.

- Due consideration must be given to minimizing noise to residents or the public.

Definitions:

AUTHORIZED LOCOMOTIVE  DRIVER: a financial member of OTHR who is a  certified driver for driving a Class 73 Diesel Locomotive. The driver must carry a current, numbered certificate (card) indicating that satisfactory qualifications are held.

OBERON STATION YARD is defined as all track located north of the buffer stops at the south end of Oberon yard but ending south of the level crossing at Albion Street. There are no road level crossings within Oberon Yard.

DANGER ZONE: everywhere within 3m horizontally from the nearest rail and any distance above or below this 3m, unless a safe place exists or can be created.