Document Status

| Version | Date | Revision | Prepared | Reviewed | Approved |
| --- | --- | --- | --- | --- | --- |
| Draft | 25 May 2012 | Rev.0 | SM | SEC, VP |  |
| A | 15/08/12 | Rev0 | SM | Mgmnt Com | 15/08/12 |

Revision Record

| Revision | Date Issued: | Description of change: |
| --- | --- | --- |
| 1 | 20/08/12 | Changes required by Committee during approval |

Introduction

Oberon Tarana Heritage Railway will be operational across level crossings between the Oberon Station and Hazelgrove. The two main Level Crossings in Albion Street and Lowes Mount Road are being rebuilt by Oberon Council with a completion date of Nov 2012.

These two crossings will be equipped with warning signs to indicate that there is a Level Crossing, before the crossing, and Give Way to Trains signs at the crossing. These signs will fold down and will only be raised and visible on days the trains are running.

Council will erect appropriate signs at all the other Level Crossings to give the same warnings to traffic, pedestrians or users of the Bike Track that runs alongside the track.

OTHR Operations

During all operations of passenger trains or track maintenance vehicles the following procedures for drivers and crew will be followed to minimise any risk to vehicles or pedestrians.

- Prior to operations a track inspection vehicle will travel along the OTHR route to inspect the track for any obstructions and to raise all the Level Crossing signage.

- This track inspection vehicle will not traverse any crossing until the warning signs are set. No train operations will occur till the track inspection vehicle has returned and signed off that the track is clear and all signs are set.

- During train operations all trains will be limited to 20kph and will stop prior to all Level Crossings that will be carrying vehicular traffic.

- The driver will sound the engine horn and proceed when all vehicles on the crossing are stopped and clear of the crossing.

- At all other crossings where there is clear visibility and there is no traffic or pedestrians the engine will proceed at 5kph through the crossing on advice from the Observer who is in the drivers cab.

- At the completion of each days train operations the Operations Manager will ensue that the final train crew has changed all the signs to indicate that the track is not being used and folded the Give Way to Train signs.