|  | PO Box 299 Oberon NSW 2787 ABN 98 107 506 208 | Version | Author | Reviewed | Reference |
| --- | --- | --- | --- | --- | --- |
|  |  | B - Rev 2 | SM | SEC/TM | TB-016 |
|  |  | 29 Oct 2009 |  |  |  |
|  |  | Working With Moving Machinery |  |  |  |

Work Area: ………………………………… 	Start Time: ………………………

Work Crew: ……………………………………………………………………………

Location: ………………………………… 		Date: …………………………….

Team Leader: …………………………… Presenter: …………………………………

Topic: Working With Moving Machinery

- No person must drive or operate any vehicle, mobile or other equipment unless they have been properly trained and authorised to do so. Where necessary the appropriate certificate or license must be held and should be shown on the operator’s Certificate of Competency ID card.

- Machinery must be shut down if you have to check, clean or carry out maintenance.

- Moving machinery has pinch points and hazardous moving parts - take care.

- Never wear loose clothing or long hair near rotating machinery eg. PTO shafts.

- If possible, avoid working near any moving machinery.

- Keep other people clear of moving machinery - watch out for your fellow workers.

- Keep unauthorised personnel clear of the work site.

- Wear appropriate PPE including hard hat if closer than 5m from the edge of the Swing Zone. 
The Swing Zone is a circular area, the radius of which is measured from the centre of the machine to the outside edge of the implement or gripped object when the machine is at full extension.

- Never work or enter the “Swing Zone”. Maintain the minimum 5m clearance at all times. Ask your supervisor if you are unsure.

- If machinery is nearby, designate a “spotter” to observe & control traffic movement.

- The spotter must be visible to the machine operator.

- If possible, erect safety barriers and warning signs around operating machinery.

- Never approach a machine operator from their blind side.

- Always gain the operators attention and ensure the machine is stopped prior to approaching the driver’s compartment.

- The machine should be operated at a speed appropriate to the work place.

- Advise the team leader if you notice faulty or damaged machinery or unsafe practices.

- Any QUESTIONS or COMMENTS? 
The Presenter must record any questions or comments on the back of this form

Please sign that you have attended and that you understood the content of this meeting:

| Name | Signature | Name | Signature |
| --- | --- | --- | --- |