|  | PO Box 299 Oberon NSW 2787 ABN 98 107 506 208 | Version | Author | Reviewed | Reference |
| --- | --- | --- | --- | --- | --- |
|  |  | A - Rev 1 | SM | SEC/TM | TB-023 |
|  |  | Jan 2010 |  |  |  |
|  |  | Working in a Dusty Environment |  |  |  |

Work Area: ………………………………… 	Start Time: ………………………

Work Crew: ……………………………………………………………………………

Location: ………………………………… 		Date: …………………………….

Team Leader: …………………………… Presenter: …………………………………

Topic: Working in a Dusty Environment

Follow Safe Procedures:

  - Do not dry sweep areas - wet down the surface

  - Wet down dusty work areas and processes

  - Display warning signs where necessary   ---------------------

  - Wear appropriate Dust Masks (below)

  - Adopt good “housekeeping” procedures

  - Limit overall exposure to dust generated during
track and infrastructure work.

- Beware of Silica Dust:

  - Silicosis is caused by breathing in dust that has silica particles in it. The very fine particles can scar your lungs making breathing difficult.

  - There is NO CURE for silicosis but it can be prevented by following the safety procedures mentioned.

  - Breaking up concrete releases lots of silica dust as can abrasive blasting and some sawing and grinding processes

- Dust Mask Standards:

  - P1 disposable dust respirators are suitable for protection against mechanically generated particles to 1 micron, such as dust and mist.

  - P2 disposable dust respirators are suitable for protection against mechanically and thermally generated particles to 0.3 micron, such as toxic dust including asbestos, mist and welding fumes.

- Wear a P2 Mask also for work such as Oxy Cutting, Fumes and Smoke, Welding, Grinding.

- Wear Eye Protection in dusty conditions.

- Any QUESTIONS or COMMENTS? 
The Presenter must record any questions or comments on the back of this form

Please sign that you have attended and that you understood the content of this meeting:

ATTENDEES:

| Name | Signature | Name | Signature |
| --- | --- | --- | --- |